Personalised Transport Security Camera

Status Lights									 (07.25)

Approved Security Camera Systems

The following security camera systems are approved by the Department of Transport and Main Roads (TMR):

- Mark 5 TaxiCam (VerifEye/Lytx)

- SnapShot Mark 4 with internal camera model PV3006-001 (MTData/MTI)

- SnapShot Mark 4 with internal camera model PV3006-002 (MTData/MTI)

- TaxicamHD Mobile CCTV System (Taxicomms)

- Streamax (G60DT) X5-Lite

- Streamax (G60DT) X5-Lite V2

- Streamax (G60DT) X5-Lite Mini

- 13CABSi (Taxitech)

- Crown Cabs Camera Mark 1

Notes:

- Only approved security cameras in a personalised transport vehicle may face inwards to record the driver or passengers while a personalised transport service is being provided.

- Externally focused forward or rear facing cameras are allowed in all personalised transport vehicles, however, these cameras must not record images or audio of persons inside the vehicle. Camera specifications and the download of images from these cameras are not regulated by TMR.

- If a security camera system is faulty, it is the operator’s responsibility to inform TMR that the system is not fully operational and to arrange for the system to be fixed or replaced. A Fault Notification Form (F4760 ) must be sent to TMR and kept in the vehicle while the system is not operational. The fault notification form is valid for four days following the latter of either the date TMR is informed or the start date provided on the notification.

The information overleaf aims to help identify the operational status of an approved personalised transport security camera system using the visual indicator lights. Further information regarding the personalised transport security camera scheme is available here.

Mark 5 TaxiCam (VerifEye/Lytx)

SnapShot Mark 4 (MTData/MTI)

The camera system is not fully operational if the visual indicator status light display is:

Verifeye				MTData/MTI

Blinking Red; or

Red and Green

Not illuminated (no light)

TaxicamHD Mobile CCTV System (Taxicomms)

The camera is working properly and recording when both the power and

recording LED lights are lit green.

When the camera is asleep the recording light will not be lit. It will take up

to 30 seconds for this to become lit after the vehicle ignition is turned on.

If no LEDs are lit the power maybe disconnected.

If an Error LED is lit RED or the Record LED is amber a fault may have occurred.

Streamax systems

Streamax (G60DT) X5-Lite

Streamax (G60DT) X5-Lite V2

Streamax (G60DT) X5-Lite Mini

Indicator Light 	Meaning

POW 			Power light

REC 			Indication that the unit is recording footage

HDD* 		Indication that the HDD is working correctly

ALM 			An alarm is triggered

VIDEO* 		Video loss (camera is no longer displaying video)

ERR			Error with the system or camera

*Applicable for older indicator only.

*For newer indicator, PWR and Green lights only indicate system working, any other lights indicate a fault.

13CABSi (Taxitech) & Crown Cabs Camera Mark 1

Indicator Light 	Meaning

POW 			The unit has power

REC 			The unit is recording footage

HDD 			The HDD is working correctly

ALM 			An alarm has been triggered

VIDEO 		Video loss (Camera has not been detected)

ERR			Error with the system, GPS or camera

Crown Cabs Camera Mark 1

Indicator Light 	Meaning

If the MDVR system is working correctly, ‘PWR’, ‘REC’ and ‘HDD’ lights should always be lit.

PWR – Green Light

REC – Orange Light

HDD – Orange Light

Camera missing (video loss) – VLOSS and ALM to stay flashing.

GPS (no signal) – ERR to stay flashing.

System fault (the cameras have stopped recording to hard disk) – ERR to stay flashing, HDD off, REC off.