| Structure Id |  |  |  |  |  |  |  | Name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8086 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crossing Name |  |  |  |  |  |  |  | Alt. Name |  |  |  |  |  |  |
| Smothering Creek |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Structure Type |  |  |  |  |  |  |  | Owner |  |  |  |  |  |  |
| Bridge |  |  |  |  |  |  |  | TMR Department of Transport & Main Roads |  |  |  |  |  |  |
| Construction Type |  |  |  |  |  |  |  | District |  |  |  |  |  |  |
| Girder/Beam |  |  |  |  |  |  |  | 12 - Wide Bay District |  |  |  |  |  |  |
| Construction Material |  |  |  |  |  |  |  | LGA Id |  |  |  |  |  |  |
| Timber |  |  |  |  |  |  |  | 228 - Fraser Coast Regional Council |  |  |  |  |  |  |
| Inspector |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |
| Evan T Matthews |  |  |  |  |  |  |  | 18/05/2015 |  |  |  |  |  |  |
| Inspection |  | Level 2 |  |  | Programmed |  |  |  |  |  |  |  |  |  |
|  |  | Level 3 |  |  | Exceptional |  |  |  |  |  | Underwater |  |  |  |
| Road Section |  |  |  |  |  |  |  | Start |  | End |  |  | TDist |  |
| Id | Description |  | S | Cway |  |  | S | RPC | Tdist | RPC |  | Tdist | Start | End |
| 487 | BROOWEENA - WO |  | C | 1 |  |  | C | 4 | 0.000 | 4 |  | 0.018 | 28.151 | 28.169 |

| Structure Id |  | Name |  |  |  | Inspection date |  |  |  | Inspection Level |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8086 |  |  |  |  |  | 18/05/2016 |  |  |  | 2 | 3 | Programmed | Exceptional |  |  |  | Underwater |  |  |  |  |
| Component Location |  |  |  |  |  | Exposure Class |  | Condition State | Description of Defects - Detailed Description - Estimated Quantity - “Other” action required - Urgency of action (what, who, when, how) - Recommended Testing - Reference of sketches and photo (Roll/Exposure Nos) |  |  |  |  |  |  |  |  | Required Action (X) |  |  |
| Modification | Group |  | Component |  | Standard Number |  |  |  |  |  |  |  |  |  |  |  |  | Monitor | Level 3 Inspection | Other |
| O | AP1 |  | AP |  | 70 O | 2 |  | 3 | AP1-AP:CS3 Cracks & bumps throughout (concrete causeway). Moderate bleeding bothe wheel paths & 30mm depressions full width at transition over cold mix repairds - Photo 4 |  |  |  |  |  |  |  |  |  |  |  |
| O | AP1 |  | GR1 |  | 72 S | 2 |  | 4 | AP1-GR1:CS4 GR not connected or to current standards. Some timber posts with termite damage have been replaced - Photo 3 (install new GR) |  |  |  |  |  |  |  |  |  |  |  |
| O | AP1 |  | GR2 |  | 72 S | 2 |  | 4 | AP1-GR2:CS4 GR mpt cpmmected pr tp cirremt stamdards. Some timber posts with termite damage have been replaced - Photo 3 (install new GR) |  |  |  |  |  |  |  |  |  |  |  |
| O | S1 |  | WS |  | 1 O | 1 |  | 3 | S1-WS - Cracking areas failing over loose deck 5 planks out from transition - Photo 5 (tighten planks/relay WS) Minor cracking throught & small depression LHS E1 against kerb. Repairs evident MS above previous deck repairs. |  |  |  |  |  |  |  |  |  |  |  |
| O | S2 |  | K1 |  | 3 T | 1 |  | 3 | S2-K1-CS3, 2.4lm LHS E2 decay & loss of section (kerb needs painting) |  |  |  |  |  |  |  |  |  |  |  |
| O | S3 |  | WS |  | 1 O | 1 |  | 3 | S3-WS:CS3 Cracking areas failing over loose deck 5 planks out from transition - Photo 6 (tighten deck/relay pavement) |  |  |  |  |  |  |  |  |  |  |  |
| O | AP2 |  | AP |  | 70 O | 2 |  | 3 | AP2-AP:CS3 Crack & bump throughout concrete causeway. Moderate bleeding both wheel paths & 40mm depression & cracking full width at transition over cold mix repairs - Photo 7 (need reprofiling) |  |  |  |  |  |  |  |  |  |  |  |
| O | AP2 |  | GR1 |  | 72 S | 2 |  | 4 | AP2-GR1:CS4 GR not connected or to current standards. Some timber posts rotted & termite affected. |  |  |  |  |  |  |  |  |  |  |  |
| O | AP2 |  | GR2 |  | 72 S | 2 |  | 4 | AP2-GR2:CS4 GR not connected or to current standards. Some timber posts rotted & termite affected. |  |  |  |  |  |  |  |  |  |  |  |
| O | A1 |  | P1 |  | 60 T | 2 |  | 4 | A1-P1:CS4, 53% consumed at top. Visible pipe & 11mm cracking - Photo 15 (replace) |  |  |  |  |  |  |  |  |  |  |  |
| O | S1 |  | D1 |  | 29 T | 1 |  | 4 | S1-D - 17 x planks have weathering & rot on end both sides, 8 LHS & 9 RHS - Photos 8 & 9 (slice in new ends) |  |  |  |  |  |  |  |  |  |  |  |
| O | S2 |  | D1 |  | 29 T | 1 |  | 4 | S2-D:CS4 17 x planks have rot on ends, 8 LHS & 9 RHS - Photos 8 & 9 indicative |  |  |  |  |  |  |  |  |  |  |  |
| O | S3 |  | D1 |  | 29 T | 1 |  | 4 | S3-D -8 x planks have weathering & rot on ends, 4 LHS & 4 RHS (splice in new ends) - Photos 8 & 9 indicative |  |  |  |  |  |  |  |  |  |  |  |
| O | A2 |  | WW1 |  | 51 O | 2 |  | 3 | A2-WW1:CS3 Wing rail replaced treated WP rotted |  |  |  |  |  |  |  |  |  |  |  |
| O | A2 |  | P1 |  | 60 T | 2 |  | 4 | A2-P1:CS4 59% consumed at top - Photo 16 (replace) |  |  |  |  |  |  |  |  |  |  |  |
| O | A2 |  | P2 |  | 60 T | 2 |  | 4 | A2-P2:CS4 Sever fire damage at top & 120mm pipe - Photo 17 (replace) |  |  |  |  |  |  |  |  |  |  |  |