| Structure ID |  |  |  |  |  |  |  |  |  | Name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8086 |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |  |  |
| Crossing Name |  |  |  |  |  |  |  |  |  | Alt. Name |  |  |  |  |  |  |  |  |  |
| Smothering Creek |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |  |  |
| Structure Type |  |  |  |  |  |  |  |  |  | Owner |  |  |  |  |  |  |  |  |  |
| Bridge |  |  |  |  |  |  |  |  |  | TMR Department of Transport & Main Road |  |  |  |  |  |  |  |  |  |
| Construction Type |  |  |  |  |  |  |  |  |  | District |  |  |  |  |  |  |  |  |  |
| Girder/Beam |  |  |  |  |  |  |  |  |  | 12 Wide Bay District |  |  |  |  |  |  |  |  |  |
| Construction Material |  |  |  |  |  |  |  |  |  | LGA Id |  |  |  |  |  |  |  |  |  |
| Timber |  |  |  |  |  |  |  |  |  | 228 Fraser Coast Regional Council |  |  |  |  |  |  |  |  |  |
| Inspector |  |  | Date |  |  |  |  |  |  | Date of Next Inspection |  |  |  |  |  | Work Item No. |  |  |  |
| Evan T Matthews |  |  | 26/06/2016 |  |  |  |  |  |  | 18/05/2017 |  |  |  |  |  | - |  |  |  |
| Inspection Level 1 |  | Programmed |  |  |  |  |  | Exceptional |  |  |  |  |  | Perm. Standing Water |  |  |  |  |  |
|  |  |  |  |  |  | Vermin Screens |  |  |  |  |  |  |  | Security Measures |  |  |  |  |  |
| Road Section |  |  |  |  |  |  | Start |  |  |  |  | End |  |  |  |  | TDist |  |  |
| Id | Description | S |  | Cway | S |  | RPC |  |  |  | Dist | RPC |  |  | Dist |  | Start | End |  |
| 487 | Brooweena – Woolooga Road | C |  | 1 | C |  | 4 |  |  |  | 0.000 | 4 |  |  | 0.018 |  | 28.151 | 28.169 |  |

| Structure Id | Name |  | Inspection date |  |  |  | Inspection Level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8086 | -- |  | 18/05/2016 |  |  |  | 1 |  | Programmed | Exceptional | Vermin Screens |  |  |  |  |  |  |  | Perm. Standing Water |  |  | Security Measures |  |
| Inspection Elements (*Refer to bottom of form) |  |  |  |  | Problem (tick) |  |  | Location and comments (include maintenance activity number) |  |  |  | Rectified |  |  |  |  | Mtce Required |  |  |  | Inspect Required |  |  |
|  |  |  |  |  | Y | N |  |  |  |  |  | Y |  |  | N |  | Y |  |  | N | Y |  | N |
| Approaches - Signs/Delineation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - completeness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose/missing fixings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - cleanliness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - orientation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Guardrail |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - correct height/alignment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose or missing fixings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - impact damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - material deterioration (steel corrosion, timber decay etc.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damaged/missing spacer blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - connection to bridge barrier (the approach barrier and bridge barrier should preferably be interconnected. If not, note in “Comments” section of report form). |  |  |  |  |  |  |  | Not all Gr are connected |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Road Drainage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - debris/vegetation growth inside drains, channels, inlet/outlet pits and sumps which may obstruct free drainage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - leaking drainage pits/structures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - scour/erosion at drainage outlets, particularly adjacent to abutments/foundations, culvert outlets and deck run-off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Wearing Surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - settlement |  |  |  |  |  |  |  | Settlement & bleeding within both approaches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - depressions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - pot holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - cracking |  |  |  |  |  |  |  | Moderate cracking over concrete joints within both approaches due to loose deck planks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface over Structure - Wearing Surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - settlement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - depressions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - pot holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - cracking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Footways (if any) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - unevenness/trip hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Bridge Barrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - correct height/alignment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose or missing fixings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - impact damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - material deterioration (steel corrosion, timber decay etc.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damaged/missing spacer blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - delineators for completeness, damage, cleanliness, orientation and visibility to road users |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Deck Joints |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose/missing fixings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damaged/missing components |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - dirt/detritus build up in joints which may impede free movement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - leakage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damage/deterioration of nosings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Embankments and Waterways - Embankments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - erosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - scour |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - slope stability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Slope/batter protection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - undermining |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - settlement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loss of material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Vegetation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - bushes, trees within 2.0 m of abutments and wingwalls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - bushes/trees within waterway channel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - vegetation affecting sight distance onto/across structure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Waterway |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - accumulation of debris against or adjacent to structure |  |  |  |  |  |  |  | Flood debris caught on piers primarily U/S |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - localised scour adjacent to/beneath the structure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - lateral bank erosion adjacent to/beneath the structure |  |  |  |  |  |  |  | Minor erosion at spans 1 & 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - channel degrading |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - channel aggrading |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - structure in permanent standing water |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substructure - Abutments, piers, wingwalls, retaining structures and foundations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - cracking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - splitting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - distortion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - movement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - steel corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - vegetation growth in joints of coursed masonry/fascia panels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - weepholes for blockages affecting free drainage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - timber members for decay, termite activity, marine borer and other insect attack |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Headstocks, bearing pedestals and substructure drains |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - accumulation of dirt and debris which may obstruct free drainage and cause ponding or restrict bearing movement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - corrosion |  |  |  |  |  |  |  | Corbels inspected instead |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - excessive deflection/bulging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - delamination of elastomeric bearings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - damage to pedestals/plinths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - noticeable build-up of deposits of aggressive salts, silt, debris and bird or bat droppings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Superstructure - Deck/Girders |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - obvious evidence of spalling, cracking, staining, dampness or corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - excessive movement/vibration under load |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - for noticeable build-up of deposits of aggressive salts, silt, debris and bird or bat droppings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - blocked vent holes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - timber members for termite activity, rotting, marine borer and other insect attack |  |  |  |  |  |  |  | All deck spans exhibit end rot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - timber members for excessive member deflections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - timber girders and corbels for excessive sniping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose joints and fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - propping for tightness of wedges in deck cambering or temporary works |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Culvert Barrel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - distortion /deflection of barrel |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - invert corrosion/abrasion |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - obvious evidence of spalling, cracking, staining, dampness or corrosion |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Large Traffic Management (LTMS) and gantries - Footings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - obvious evidence of spalling, cracking or reinforcement corrosion |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - rotation/settlement |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - erosion/undermining |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Base Plates, Fittings and Hold-Down Bolts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - cracking, spalling or voids in mortar pad |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - debris/fill over base plate |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - corrosion of fixings/base plate |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose/missing fixings and thread engagement |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Corrosion, buckling, bending, rupture, rotation or misalignment of sections |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Impact damage |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Verticality of members |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Protective coating loss |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Loose/missing fixings and thread engagement |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cantilever arms/gantry beams |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - corrosion, buckling, bending, rupture, rotation or misalignment of sections |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - separation/distortion at joints/splices |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - sagging of members |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - protective coating loss |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - loose/missing fixings and thread engagement |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Signage and Ancillaries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Completeness, damage, cleanliness, orientation and visibility to road users |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous - Roadway Beneath Structure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - delineation |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - barriers |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - road drainage |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - location and condition of any services attached to or in close proximity to the structure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Appearance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - graffiti |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Material * | Defects Description |
| --- | --- |
| Concrete | Cracking, spalling, corrosion of reinforcement, drummy areas |
| Steel | Bending, buckling, cracking, distortion, loose bolts, rivets, corrosion, coating damage |
| Timber | Splitting, crushing, decay, infestation, loose bolts or pins |
| Masonry | Cracking, opening joints, mortar loss, bulging |
| Bituminous Surfacing | Cracking, crazing, breaking up, lifting off, rutting, pushing |
| Protective Coatings | Cracked, peeling, weathered |
| ** Steel Culverts | Probe or sound culvert walls at normal water level, check for pitting or loss of culvert material |

General Comments Rebhab. works completed in 2014.

Vermin Screen Comments

Security Measures Comments