Checklist – CAC022M
Concrete Kerbs, Kerb Channel, Crossing and Dykes (MRTS03)

| Contractor |  | Date |  | Review No. |  |
| --- | --- | --- | --- | --- | --- |
| Contract No. |  | Project No. |  | Project Name |  |

| Reference | Requirements | Addressed | Comments/Observations |
| --- | --- | --- | --- |
| MRTS03 Cl 20.3 & 6.1 Cl 20.2 | Cast in Place Does the concrete work comply with the requirements of Clause 6.1 (Concrete Work) in MRTS03? Is there compressible packing in the expansion joints? Is the compressible packaging bitumen impregnated fibre board or other approved packaging? Has the insitu material in the excavation been compacted in accordance with MRTS04? Has the ground surface been moistened just prior to placing concrete? Have expansion joints been installed at regular intervals not exceeding 20 m? Have contraction joints between expansion joints been installed at regular intervals not exceeding 5 m? Has concrete been cured for a period not less than seven days before any other roadworks operations are carried out adjacent to the kerb, channel or kerb and channel? |  |  |
| MRTS03 Cl 21.2 & 6.6 Cl 21.3 | Precast Are precast concrete kerb blocks and channel blocks proprietary products? Has epoxy adhesion agent been employed, where kerb, channel and kerb and channel blocks are to be fixed to the surface of asphalt or concrete? Have kerb, channel and kerb and channel blocks been laid on bedding with minimum 50 mm thickness? Have joints (width of joints being between 7 mm and 13 mm) been finished to give smooth surfaces uniform with the surfaces of the block? Has it been ensured that there are no adjacent roadworks operations within 48 hours? Has the insitu material in the excavation been compacted in accordance with MRTS04? |  |  |
| MRTS03 Cl 20.5 | Water testing (where longitudinal grading is less than 1%) Has it been ensured that water is not ponding to a depth or more than 5 mm in any section of the channel? |  |  |
| MRTS04 Cl 5.4.2 & 5.4.3 Annexure MRS04.1 | Compliance testing Have the geometric tolerances been checked at the points nominated in the specification? Is the compaction standard for each lot represented by the characteristic value of the RDDs? |  |  |

Additional issues for consideration

|  | Is the kerbing machine suitable for the proposed mix? |
| --- | --- |

Delete below section if not required

| Audited by: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name |  | Signature |  | Date |  |