Checklist – CAC025M
Installation of Concrete Slab-Linked Box Culvert (MRTS03)

| Contractor |  | Date |  | Review No. |  |
| --- | --- | --- | --- | --- | --- |
| Contract No. |  | Project No. |  | Project Name |  |

| Reference | Requirements | Addressed | Comments/Observations |
| --- | --- | --- | --- |
| MRTS03 Cl 6.1 | Do construction requirements meet those of Cl 6.1: concrete strengths construction joints weepholes with a 300 x 300 x 150 no-fines block 100 um polythene separator sheet to foundation less than 30% rock plums in unreinforced work, and curing of concrete (seven days or as specified). |  |  |
| SD 1316,1317, 1318, 1320 | Has the base slab been constructed in accordance with the relevant standard drawings? See also concrete operations checklist. This is the critical part of the process for this type of culvert. |  |  |
| SD 1316 | Have the Crown units been installed in accordance with the relevant standard drawing? |  |  |
| MRTS03 Cl 12.3.6 | Has the contractor installed dowel bars in the top of the units to locate the spanning slabs? |  |  |
| MRTS03 Cl 12.3.6 | In multiple cell culverts are the gaps between lines of installed cells a min of 40 mm wide and: - filled with concrete end plugs extending a minimum of 250 mm along the gap (full height)? gaps between the cells are dry packed with concrete infill material (where applicable). |  |  |
| MRTS03 Cl 12.3.6 | Has the contractor filled the space between the spanning slabs with lean mix concrete? |  |  |
| MRTS03 Cl 12.3.6 | Has 24 hours elapsed prior to the placement of any other fill over the culvert? |  |  |
| MRTS03 Cl 18.3 & 19.3 | Have the end structures been constructed in accordance with the relevant Standard Drawings? |  |  |

Additional issues for consideration

|  | Additional care is required when setting our culverts are skewed and on curved alignment |
| --- | --- |

Delete below section if not required

| Audited by: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name |  | Signature |  | Date |  |