Checklist – CAC041M
Review of Side-track, Detour and Crossover (MRTS02, MUTCD Part 3 and QGTTM)

| Contractor |  | Date |  | Review No. |  |
| --- | --- | --- | --- | --- | --- |
| Contract No. |  | Project No. |  | Project Name |  |

| Reference | Requirements | Addressed | Comments / Observations |
| --- | --- | --- | --- |
| MRTS02 Clause 6.5.3.1 | Has the Contractor arranged its construction program and sequencing, so traffic flow is maintained through the works in accordance with QGTTM Part 2 and Part 3, as supplemented or amended by Clause 5.6 of Annexure MRTS02.1? |  |  |
| MRTS02 Clause 6.5.3.2 | Has the Contractor liaised and made all necessary arrangements with the relevant local government and/or other authorities concerned when implementing a detour on to existing roads? |  |  |
|  | Has the Contractor: - inspected the route for adequacy for the entire length of the detour? - implemented any parking, or other restrictions, to allow the suitable flow of detoured traffic? - provided suitable directional signage and other infrastructure to guide motorists? - performed any necessary restoration work following cessation of the detour to the approval of the relevant Authorities? |  |  |
| MRTS02 Clause 6.5.3.3 Side-tracks | Have all the following items been addressed: - design and construction in accordance with MUTCD Part 3, the QGTTM and comply with relevant Technical Specifications - location and route in accordance with Clause 5.7 of Annexure MRTS02.1 and/or shown on the drawings - surface to be cleared, grubbed and stripped for surface formation and objects within three metres removed, shielded or delineated - alignment to be aligned, formed, graded, drained and maintained to provide for safe and comfortable passage of vehicles - crossfall not more than 4% - surface as stated in Clause 5.7 of Annexure MRTS02.1, or the QGTTM Part 3 - geometric requirements as stated in Clause 5.7 of Annexure MRTS02.1 - width as stated in Clause 5.7 of Annexure MRTS02.1 - if required and not provided elsewhere in the Contract, the waterway crossings formed and designed through appropriate risk assessment and approved by the Principal. - side-tracks signed and delineated to ensure clarity of the route - lighting to comply with MRTS02.1 Clause 5.7 - where reused, temporary pavement marking to be updated and/or removed - maintained to the standard to which they were built - decommissioning, completely removed and rehabilitated to a state equivalent to that existing before the side-track was constructed. |  |  |

Additional issues for consideration

| 1 | Temporary speed limits shall be as per the QGTTM Part 3 Clause 5.5.1. |
| --- | --- |
| 2 | Side-track should be inspected periodically at night to check effectiveness of delineation and retro-reflectivity of signage. |

Note:
MRTS02 and MRTS02.1 contain references to other technical documents published by the department. Ensure that the TMD has referenced these documents in the preparation of the TMP and associated TGS.

Delete below section if not required

| Reviewed by: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name |  | Signature |  | Date |  |