Checklist – CAC057M
Erosion and Sediment Control Plan (MRTS52)

| Contractor |  | Date |  | Review No. |  |
| --- | --- | --- | --- | --- | --- |
| Contract No. |  | Project No. |  | Project Name |  |

Legend:  C Conformance  NC Non-conformance  O Observation  N/A Not Applicable  NFV  Not Fully Verifiable

This checklist is designed to assist the Contract Administrator to determine if a submitted Erosion and Sediment Control Plan (ESCP) should be deemed suitable. For high-risk projects, the Contractor is required to obtain and submit an independent assessment of their ESCP by an appropriately qualified person. The independent assessment report would fulfil the requirements of this ESCP checklist.

| Reference | Requirements | Addressed | Comments / Observations |
| --- | --- | --- | --- |
| Clause 6.2 ESCP (a) | Does the plan specify the area and Works that the plan is applicable for? Is this area appropriate? That is, does the plan cover all proposed exposed areas including full extent of cleared areas, stockpiles, Site compound / storage areas and side tracks? |  | For low-risk sites, one drawing with associated technical notes on timing of installation of controls and reference to standard drawings may be sufficient. For high risk and large scale projects the ESCP is likely to consist of multiple sets of drawings for various areas and stages of cut / fill for project. |
| Clause 6.2 ESCP (b) | Major features Are major features shown on the plan (dispersive soils, waterways)? |  |  |
| Clause 6.2 ESCP (c) | Flow direction / Contour lines Does the plan identify the direction of flow of water through contour lines or other? |  |  |
| Clause 6.2 ESCP (d) | Are the type and location of control measures clearly identified? |  |  |
| Clause 6.2 ESCP (d i) | Erosion Control Measures Does the plan identify methods to minimise soil exposure such as soil binders, geofabric, hydro mulching and early revegetation? Are these controls suitable? This can also be addressed by methods such as: - undertaking Works in key risk areas at time of year when rainfall erosivity is the lowest, and - clearing of vegetation only undertaken immediately prior to Works being actively worked. |  | This requirement may be met by statements or notes such as ‘all areas not worked for 2 weeks to be treated with PAM’. |
| Clause 6.2 ESCP (d ii) Clause 1.4 | Drainage Is clean stormwater diverted around or through the Site? Will manner of diversion keep stormwater clean (that is, not cause erosion)? |  | Evidence: Plan may need to show catchments, drainage lines to assist in determining. For high risk sites, calculation for size of drainage lines should be provided. |
| Clause 10.2 | Do all areas of the Site subject to concentrated stormwater flows, have drainage controls which have been designed to meet MRTS52 requirements? |  | Calculations required as evidence. Drainage to be designed as per Table 10.2 in MRTS52. |
| Clause 6.2 ESCP (d iii) | Sediment Controls For medium and high risk sites, does the plan show the runoff from all areas which are not effectively stabilised draining to sediment basins? Sediment basins recommended for projects where catchments and exposed soil >10000 m2 and soil loss >150 t/ha/year. |  | If answer is no, are there additional controls for areas that do not drain to sediment basin? |
| Clause 6.2 ESCP (d iii) | For low risk sites, does the plan show the runoff from all areas which are not effectively stabilised draining to suitable Type 2 or Type 3 controls? |  | Refer to International Erosion Control Association (IECA) ‘Sediment Control Classification System’ design guideline for further information. |
| Clause 10.2 | Is each sediment basin designed with appropriate capacity and structural strength such that: - it has the capacity to contain all the stormwater runoff from at least the 80th percentile 5 day rainfall event; and - sediment basin inlets address MRTS52 Table 8.2, and - sediment basin emergency outlet and embankments address MRTS52 Table 10.2. |  | Evidence: Calculation sheets. |
| Clause 6.2 ESCP (d iv) | Are sediment basin / stormwater discharge points nominated? Are they suitable? |  | The discharge point must be stable so that erosion does not occur. |
| Clause 6.2 ESCP (d v) Clause 8.1.4 | Site Exit Points Have Site access points been identified and appropriate controls specified? |  | All exit points to bitumen roadway should have wheel wash. |
| Clause 6.2 ESCP | Timing / staging of controls Does the plan identify the installation sequence and timing of installation of controls? |  | For low risk projects, detailed timing not required - MRTS52 specification requires that all controls to be installed as soon as possible. |
| Clause 6.2 | Is the ESCP site specific and does it consider the following: |  |  |
|  | - Seasonal conditions. |  | Works should not occur where monthly R factor> 1500. Where R factor > 285, ensure drainage controls sized for intensive rainfall events and protective material readily available on-site / on short notice. |
|  | - Soil Type – particularly dispersive soils, sodic, saline soils, and acid sulfate soils. |  | Examples of appropriate design for dispersive include: treat soil particularly if ESP > 6% minimise cut into soil, for example, use catch banks or berms, rather than catch drains to divert water, ensure all temporary drains are spoon rather than v-drains disperse water rather than concentrating flows. |
|  | - Topography – particularly areas with; natural, during construction, or final slope, that is > 10%. |  | Where slope factor risks are significant, need to ensure suitable controls to reduce flow velocity. Energy dissipaters should be specified for end of batter chutes. |
|  | - Local hydrology and drainage affecting the worksite, including temporary and overland flow paths. |  |  |
|  | - Specific project issues. |  | If a waterway runs through Site or is directly adjacent, maintain buffer zones as long as possible for clean water separation. Consider protected flora and fauna, species breeding times, and project approval conditions. |
| Monitoring and Management |  |  |  |
| Clause 6.2 ESCP (g) | Is there a contingency plan for severe wet weather event to minimise likelihood of failure of ESC measures? |  | These may be included in the project Severe Weather Management Plan (SWMP) or ESCP. |
| Clause 6.2 ESCP (h) | Personnel Have the ESCP(S) been prepared and signed off by person with required experience? |  | Requirement is: Low risk – person with training as environmental representative and has at least 5 years' experience in relevant construction type. General – Appropriately Qualified Person (AQP) with experience in relevant construction type. High Risk – AQP to develop, plus reviewed by an independent AQP, both with experience in relevant construction type. |

| Recommendation |  |
| --- | --- |
|  | The ESCP is suitable as an overarching ESCP. Progressive ESCP sub-plans detailing specific erosion and sediment controls for particular Work areas, are required to be submitted prior to work in each area. |
|  | The ESCP is considered suitable for <insert area / Works and so on>. It is recommended that it be assessed as suitable by the Administrator. |
|  | The ESCP is not considered suitable. Works may not commence until the ESCP is revised, resubmitted and assessed as suitable by the Administrator. |

| Reviewed by: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name |  | Signature |  | Date |  |

EMP(C) checklist (CAC003M) is suitable for low-risk projects otherwise this checklist should be used.

This checklist has been based upon MRTS52, checklist CAC003M, IECA checklist for ESCP, and the checklists within the Department of Environment, Tourism, Science and Innovation (DETSI) Procedural Guides: Releases to waters from building sites and small construction sites (less than 2500m2), and Releases to waters from land development sites and construction sites 2500m2 and greater.