Checklist – CAC062M

MRTS72 Manufacture of Precast Concrete Elements incorporating MRTS70 Concrete and MRTS71 Reinforcing Steel

This checklist can be used for both special class and normal class precast concrete elements. Exclusions for normal class elements are marked (*).

| Contractor |  | Date |  | Review No. |  |
| --- | --- | --- | --- | --- | --- |
| Contract No. |  | Project No. |  | Project Name |  |
| Unit Numbers |  | Unit Types |  | Date(s) Manufactured |  |

| Reference | Documentation / Inspection | Addressed | Comments/Observations |
| --- | --- | --- | --- |
| MRTS72 Clause 5.3 | HP1 New manufacturing procedure - Check with Structures Construction Materials if innovative procedure is proposed |  |  |
| MRTS72 Clause 5.4 | HP2 Product and supplier details - Drawings (no reinforcement detail required) - Quality plan - Precast registration certificate - Design approval (if applicable) - Casting programme - MRTS70 (HP8) - Mix design approval certificate (*) - Project-specific requirements - MRTS71 (HP1) - Reinforcing supplier’s registration certificate - MRTS71 (HP7) - Welding procedure specification - MRTS71 (HP4) - Certified lifting diagram for large reinforcing cage |  |  |
| MRTS72 Clause 5.4.1 | HP3 Sample Panel - Visual inspection |  |  |
| MRTS71 Clauses 5-7 | Steel reinforcement - Delivery dockets / bundle tags - Registered supplier - Grade and ductility class - Visual inspection - Surface condition - Storage - Shape/Size - Welding |  |  |
| MRTS72 Clause 9.2 MRTS70 Clause 16.3 | WP1 Placement of concrete - Pre-pour visual inspection - Reinforcement position - Cover-to-reinforcement / spacers - No foreign material - Thermocouples (if temperature monitoring is required by MRTS70) - Electrical conductivity of reinforcement (if required) - Lifting anchor position - Formwork compliant |  |  |
| MRTS70 Clause 11 Clause 16.4 | Concrete Delivery - Delivery docket (approved mix (*)) - Field sheet - Slump/spread result (*) - Batch/arrival times - Concrete temperature - Ambient conditions - Visual inspection - No surprises |  |  |
| MRTS70 Clause 16.5 | Placement and Compaction - Visual Inspection - Concrete - Placement - Compaction - Evaporation retardation - Curing |  |  |
| MRTS72 Clause 9.7 Clause 9.8 Clause 10.1 Clause 11 | WP2 Product Inspection - Visual Inspection - Surface finish - Defects - Dimensions - Cracking - Product marking |  |  |
| MRTS72 Clause 11 | HP4 Material Data Record - Concrete strength report (NATA-endorsed) (*) - Lot register - As-constructed dimensions - Heat-accelerated curing charts |  |  |

Additional items for consideration

Delete below section if not required

| Audited by: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Name |  | Signature |  | Date |  |