| Annexure MRTS05.1 (July 2022) |  |  |  |
| --- | --- | --- | --- |
| Unbound Pavements |  |  |  |
| Specific Contract Requirements |  |  |  |
| Contract Number |  |  |  |
| Note: | Clause references within brackets in this Annexure refer to Clauses in the parent Technical Specification MRTS05 unless otherwise noted. |  |  |

| # Material compliance testing locations (Clause 9.2) |  |
| --- | --- |
|  | For the following locations and/or material subtypes, material compliance testing shall be undertaken on samples taken from the pavement. If no locations or material subtypes are given, all samples shall be taken from the stockpile. |

| # Material requirements ## Pavement material locations (Clauses 7.1.1, 7.2.1, 7.3.1 and 8.1) |  |  |
| --- | --- | --- |
|  | The following pavement material subtypes shall be used in the locations stated. |  |
| Location |  | Material Subtype |

| ## Design Traffic (Clause 7.2.1) |  |  |
| --- | --- | --- |
| Location |  | Average Daily Traffic in the Design Lane in Year of Opening – Equivalent Standard Axles (ESA) |
| If no indication is given for a particular location, the Administrator shall confirm the average daily traffic in the design lane in year of opening based on advice from the project's pavement designer. |  |  |
| ## Type 4 material requirements ### Additional requirements for Type 4 material (Clause 7.4) |  |  |
|  | The following additional requirements shall apply to Type 4 materials. |  |

To utilise a Type 4 material, the Designer must develop the standards and requirements relevant to the particular material and include these in this Clause. As a minimum this would typically include a particle size distribution and linear shrinkage requirement.

| ### CBR moisture content for Type 4 material (Clauses 4.1.2 and 7.4) |  |  |
| --- | --- | --- |
|  | The relative moisture content for reporting CBR test results for Type 4 material is given below. |  |
| Material Subtype |  | Relative Moisture Content |
| If no indication is given for a particular material subtype, the CBR shall be reported at the Optimum Moisture Content (OMC). |  |  |

| ## Material stockpiles (Clause 7.7) |  |
| --- | --- |
|  | Stockpiles of pavement material shall be located as stated below. If no indication is given, material stockpiles shall be located as stated in Clause 7.7. |

| # Construction requirements ## Construction equipment ### Paving equipment – Type 2, 3 or 4 materials (Clause 8.3.1.1.3) |  |
| --- | --- |
|  | For pavement constructed using Type 2, 3 and 4 materials, the following paving equipment shall be used. If no indication is given, the paving equipment to be used shall be nominated by the Contractor in their Unbound Pavement Construction Procedure. |
|  | The above requirements are not applicable to Type 1 (HSG) materials. |

| ### Intelligent Construction Rollers (Clause 8.3.1.3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Intelligent Construction (IC) rollers shall be used to compact unbound pavement material | Yes |  | No |  |
|  | If no indication is given, the rollers to be used shall be nominated by the Contractor in their Unbound Pavement Construction Procedure. |  |  |  |  |

Where IC equipped rollers are not available locally, existing roller may need to be retrofitted with IC equipment.

| ## Geometrics ### Vertical tolerances – Types 2, 3, and 4 materials only (Clause 8.4.4.3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | The vertical tolerance on any layer shall be alternative | A (10/15 mm) |  | B (25 mm) |  |
|  | If no indication is given, the vertical tolerance shall be Alternative A. |  |  |  |  |

| ### Deviation from a straight-edge – Types 2, 3 and 4 materials only (Clause 8.4.5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | The maximum deviation from a straight-edge shall be alternative | C (5 mm) |  | D (8 mm) |  |
|  | If no indication is given, the maximum deviation from a straight-edge shall be Alternative C. |  |  |  |  |

| ### Crossfall and straight-edge tolerances applicable on layers other than the final layer (Clauses 8.4.4.4 and 8.4.5) |  |
| --- | --- |
|  | In addition to the final layer of unbound pavement, the specified requirement for crossfall and deviation from a straight-edge shall also apply to the following layers. |
|  | If no indication is given, the requirements for crossfall and deviation from a straight-edge shall apply to the final (uppermost) layer of unbound pavement only. |

| ### Road roughness (Clause 8.4.6) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | The maximum road roughness shall be (Rs) |  |  | m/km |
|  | If no indication is given, the requirement shall be 2.31. |  |  |  |

| # Supplementary requirements (Clauses 8.4.6 and 10) |  |
| --- | --- |
|  | The following supplementary requirements shall apply. |