| Annexure MRTS13.1 (November 2025) |  |  |  |
| --- | --- | --- | --- |
| Microsurfacing |  |  |  |
| Specific Contract Requirements |  |  |  |
| Contract Number |  |  |  |
| Note: | Clause references within brackets in this Annexure refer to Clauses in the parent Technical Specification MRTS13 unless otherwise noted. |  |  |

| # Scope and program of work (Clauses 1 and 5) The specific treatment(s) for work under this Contract shall be as stated below. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference Location (Road Number) |  |  |  |  |  |  |
| Section | From |  |  |  |  |  |
|  | To |  |  |  |  |  |
|  | Length (m) |  |  |  |  |  |
|  | Nominal Width (m) |  |  |  |  |  |
|  | Area (m²) |  |  |  |  |  |
| Microsurfacing | Mix Size (mm) |  |  |  |  |  |
|  | Thickness (mm) |  |  |  |  |  |
|  | Volume (m³) |  |  |  |  |  |
| Microsurfacing rut filling | Is rut filling required? (Yes/No) |  |  |  |  |  |
|  | Mix Size (mm) |  |  |  |  |  |
|  | Volume (m³) |  |  |  |  |  |
| Tack Coat | Is tack coating required? (Yes/No) |  |  |  |  |  |
|  | Application Rate (litres/m²) |  |  |  |  |  |

A nominal thickness of 10 mm is typically specified for Size 7 mixes and 15 mm is typically specified for Size 10 mixes. When scoping the work, designers should use the following table to determine whether a rut filling course is required. Nominal size; Size 7; Size 10. For rut filling 10 – 15 mm deep. For rut filling 15 – 25 mm deep; (see note below). Note: Application in multiple layers is suggested where the depth of rutting exceeds 2 – 2.5 times the nominal size of the mix. A rut filling course does not typically need to be included in the scope of works where the rut depths are less than 10 mm.

| # Field application (Clause 9) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Placement trial required |  | YES | ☐ | NO | ☐ |
| # Supplementary Requirements (Clause 11) The following supplementary requirements shall apply. |  |  |  |  |  |